## COLLECTION OF IECEx/ExTAG DECISIONS

| Subject: CTI Testing Status of document: Approved | Key words: - procedures - manufacturers - testing | ### Decision of Denver meeting Date: 070925 Propos: Chairman Panel of Assessors and subsequent revision by AU |
| --- | --- | --- |
| Document history: Rev. of ExTAG DS 2006/001 |  |  |

Question:   Many of the standards have a requirement for insulating material to meet a certain comparative tracking index (CTI).  Often this is in relation to minimum creepage distances.  Are ExTLs expected to test the material? If not what approach should they take to satisfy themselves about the value to be used?

Background:

ExTLs must be able to satisfy themselves beyond reasonable doubt of the actual CTI value.

To determine Comparative Tracking Index the following approaches, in order of rigour and acceptance, are known to exist:

- Testing of the actual material or test sample of the material by the ExTL itself

- Sub-contracting the testing to, or accepting data from, another ExTL

- Sub-contracting the testing to, or accepting data from, an IECEE CB Scheme CBTL that has been accepted into the CB Scheme in accordance with Clause 5.3 of IECEE 02.

- Sub-contracting the testing to another laboratory whose competence has been determined by the ExTL and associated ExCB in accordance with their procedures

- Accepting a certificate or report from another laboratory that has been assessed as competent as in 4. above (without subcontracting)

- Accepting data that has been provided by the manufacturer of the component

- Accepting data that has been provided by the manufacturer of the material

- Accepting a certificate that has been provided by the applicant

- Accepting data that has been provided by the applicant.

At present the various ExTLs in the Scheme are applying the above approaches quite differently.  This varies from testing everything in the ExTL itself to never testing and only accepting data.  Hence,  ExTAG has determined the need for this decision sheet to define  the minimum acceptable approach when producing ExTRs. ExTLs will always have the right to apply testing if they wish.

Typical standards that use the requirement for CTI include:

IEC 60079-1		Flameproof enclosures ‘d’

IEC 60079-2		Pressurised enclosures  ‘p’

IEC 60079-5		Powder filling ‘q’

IEC 60079-7 	Increased safety e

IEC 60079-11	Intrinsic safety i

IEC 60079-15	Non-sparking n

IEC 62013-1		Cap lamps

The requirements for CTI testing are included in:

IEC 60112, Method for the determination of the proof and the comparative tracking indices of solid insulating materials

Answer:

The following approach shall be used when determining Comparative Tracking Index (CTI):

| Situation |  | Minimum Acceptable Approaches | Examples |
| --- | --- | --- | --- |
| 1. | Where the CTI is fundamental to the protection technique | 1 to 4 | Ex e Ex n (except as shown below) |
| 2. | Where there is well demonstrated adherence to industry standards or where the CTI requirements are ancillary to the prime protection technique | 5 to 7 | Circuit boards used in Ex i or Ex n, or CTI for Ex d enclosures |

Approaches No. 8 and 9 shall not be used.

--------------------------