- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE:  Proposed revision of Draft Decision Sheet DS 2015/002 - Determination of faults

INTRODUCTION

This document contains remarks received from IEC Maintenance Team MT 60079-18 concerning DS 2015/002 and is presented for consideration during the 2015 ExTAG Christchurch meeting.

Members are requested to consider the proposal ahead of the ExTAG meeting.

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-18:2009 (Edition 3.0) IEC 60079-18:2004 (Second edition) | Clause: 7.2 7.2 |  |
| --- | --- | --- |
| Subject: Determination of faults Status of document: Approved | Key words: - Faults - Non-isolating components - Isolating components - Separation distances - Countable faults - Non-countable faults - Infallible separation - Infallible insulation - Infallible components - Infallible segregation | - Date: 2015 02 11 Originator of proposal: FM, SGS BASEEFA TC/SC involved: |
| Question: Regarding the application of faults as part of “ma” or “mb” evaluations, what are suitable faults that can be applied? Answer: When determining faults as part of “ma” and “mb” evaluations, there are three areas of consideration… - Non-isolating components (7.2.2) - Isolating components (7.2.3) - Separation distances (7.2.4) When considering faults involving these areas, there are five key “terms”… - Countable faults (3.12) - Non-countable faults (3.14) - Infallible separation or insulation (3.13) - Infallible components (7.2.2) - Infallible segregation (7.2.3) Based on the above, when determining whether separation distances are to be subjected to faults, the following consideration applies according to 7.2.4… - Countable faults: Separation distances can be faulted - - only one location for “mb”, or two locations “ma” - - if the distances are between a) the minimum distances given for level of protection “mc” in Table 1 and b) the minimum distances given for level of protection “ma” and “mb” in Table 1. - Non-countable faults: Separation distances can be faulted - - any number of locations for either “mb” or “ma” - - if the distances are less than the minimum distances given for level of protection “mc” in Table 1. - Infallible separation distances: Separation distances cannot be faulted - - neither for “mb” nor “ma” - - if the distances comply with the minimum distances given for level of protection “ma” and “mb” in Table 1. Note: For separation distances involving solid insulation, they cannot be faulted if the distance through solid insulation is at least 0,1 mm and complies with the dielectric strength test of 8.2.4. Further, based on the above, when determining whether non-isolating components are to be subjected to faults, the following consideration applies according to 7.2.2... - Countable faults: There are no countable component faults. - Non-countable faults: Any component can be faulted - - any number of components for either “mb” or “ma” - - if the components do not comply with 7.2.2. - Infallible components: Components cannot be faulted - - neither for “mb” nor “ma” - - if the components comply with 7.2.2. And finally, based on the above, when determining whether the segregation provided by isolating components is to be subjected to faults, the following consideration applies according to 7.2.3... - Countable faults: There are no countable segregation faults. - Non-countable faults: Any segregation provided by isolating components can be faulted - - any number of segregations for either “mb” or “ma” - - if the segregation does not comply with 7.2.3. - Infallible components: Segregation provided by isolating components cannot be faulted - - neither for “mb” nor “ma” - - if the segregation complies with 7.2.3. Exception: In the case of infallible transformers as defined in IEC 60079-11 this only applies for those transformers meeting the requirements for ‘ia’ or ‘ib’. |  |  |