- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM

FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR

USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – Scope exception regarding LED light sources

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document ExTAG/373 /CD Draft ExTAG Decision Sheet – Scope exception regarding LED light sources has been prepared by UL LLC with involvement from the following, IEC/TC 31 MT 60079-28, and is issued for comment.

Please submit comments using the attached comments table by -

2015 07 27 to

Christine Kane

In the absence of comments the document will proceed for publication.

On behalf of

Mr. Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observations to be completed by the originator |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-28:2015 (Edition 2.0) IEC 60079-28:2006 (First Edition), including I-SH 01 | Clause: 1 1 |  |
| --- | --- | --- |
| Subject: Scope exception regarding LED light sources Status of document: Draft | Key words: - Optical radiation - Light-emitting diode - LED - Luminaire | - Date: 2015 06 22 Originator of proposal: UL LLC TC/SC involved: IEC/TC 31 MT 60079-28 |
| Question Are all LED light sources included within the scope of IEC 60079-28? Answer No. Regarding continuous LED light sources for EPLs Ga, Gb, Da and/or Db, these are included within the scope of IEC 60079-28. However, continuous LED light sources for EPLs Gc and Dc are not included within the scope. Regarding pulsed LED light sources for all EPLs, these are also included within the scope of IEC 60079-28. In accordance with Item 2) under the paragraph that details Scope exceptions, all LED light sources are included within the scope of IEC 60079-28 except for continuous divergent LED light sources in EPL Gc or Dc applications. The Note under this Scope exception provides informative content regarding the reasoning for this limited Scope exception at this time regarding LED light sources. Consideration should also be given to the other Scope exceptions when determining if a given application of an LED light source is covered under the Scope of IEC 60079-28. |  |  |