COLLECTION OF IECEx / ExTAG DECISIONS

| Standard(s) IEC 61241-0:2004 IEC 61214.1.1:1999 IEC 60079-0:2004 IEC 60079-0:2000 | Clause(s): 6.1.2 d), 6.1.4.1 5.1.4.1 7.1.1 7.1.2 d) 7.1.2 d) |  |
| --- | --- | --- |
| Subject: Non-metallic parts of enclosures Status of Document: Approved | Keywords - elastomeric - O-rings | Decision of meeting Date: 12 October 2004 Originator of proposal: AU |

Question: Temperature Index (TI) for plastic materials

The above clauses require the Temperature Index (TI) for plastic materials. Does this apply to O-rings and elastomeric sealing materials of enclosures?

Answer:

These clauses do not apply to O-rings and elastomeric sealing materials of enclosures. These clauses apply to structural components of enclosures e.g. enclosure material (plastic) and polycarbonate windows. However, the thermal endurance tests shall apply to O-rings and elastomeric sealing materials of enclosures if the type of protection depends on it.