INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR

CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE:	Draft ExTAG Decision Sheet – Preparation of Flameproof, “dc”, and Enclosed-break, nC”, samples

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document ExTAG/368 /CD Draft ExTAG Decision Sheet Preparation of Flameproof, “dc”, and Enclosed-break, nC”, samples – has been prepared by

UL LLC with involvement from the following, IEC/TC 31 MT 60079-1, MT 60079-15.

Please submit comments using the attached comments table by -

2015 06 29 to

Christine Kane

In the absence of comments the document will proceed for publication.

On behalf of Mr. Gauthier

- Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: Level 33, Australia Square 264 George Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 46 28 4690 Fax: +61 2 46 27 5285 e-mail: chris.agius@iecex.com http://www.iecex.com |
| --- | --- |

| Member Body | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-1:2014 (Edition 7.0) IEC 60079-15:2010 (Edition 4.0) IEC 60079-15:2005 (Third edition) | Clause: 15.5.2 22.4.1 33.4.1 |  |
| --- | --- | --- |
| Subject: Preparation of Flameproof, “dc”, and Enclosed-break, nC”, samples Status of document: Draft | Key words: - Flameproof, “dc” - Enclosed-break, “nC” - Preparation of samples - Enclosure - Thermal endurance - Tests of enclosures | - Date: 2015 05 07 Originator of proposal: UL LLC TC/SC involved: IEC/TC 31 MT 60079-1, MT 60079-15 |
| Background: Under the “preparation of samples” clause for both Flameproof, “dc”, and for Enclosed-break, “nC”, there is a Note indicating that non-metallic parts of the enclosure will have been subjected to thermal endurance testing. Question Consider a Flameproof, “dc”, device or an Enclosed-break, “nC”, device with the following characteristics: - Device incorporates a non-metallic body that houses the contacts. - Body of the device is to be fully housed within an overall enclosure in the end-application (therefore not exposed to any external mechanical abuses). For such a device construction, is the device required to be subjected to thermal endurance testing prior to the ignition test? Answers: Yes. As the body of the device is non-metallic, thermal endurance testing is required prior to the ignition testing because the type of protection depends upon the non-metallic body of the device for compliance with the ignition tests. Please note that if the body of the device also serves as an overall enclosure in the end-application, then the body also shall be subjected to the complete Tests of enclosures in accordance with IEC 60079-0. |  |  |