- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – Card Edge Connectors

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This ExTAG Draft Decision Sheet was prepared by SGS BASEEFA and is issued for a 4 week comment period.

Please submit comments using the attached comments table by -

2015 04 14 to

Christine Kane

If no comments are received the document will proceed for publication.

On behalf of Mr. Gauthier

- Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| Member Body | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-7:2006 IEC 60079-15:2010 | Clause: 4.2.3.4 7.3.5 | - Date: 2014-03-10 Draft Originator of proposal: SGS Baseefa |
| --- | --- | --- |
| Subject: Card Edge Connectors Status of document: Draft | Key words: - PWB - Edge Connector | TC/SC involved: MT 60079-7 MT 60079-15 |
| Background: Clause 4.2.3.4 of IEC 60079-7: 2006 and clause 7.3.5 of IEC 60079-15: 2010 both cite ‘card edge connectors’ as typical examples of a pluggable connection in the notes associated with those clauses. However, clause 4.2.1 of IEC 60079 7: 2006 (Electrical connections, General) states: ‘Each type shall, as applicable: f) except as permitted by the earth continuity test of IEC 60079-0, provide contact pressure that is not applied through insulating materials’ Since the card edge, in this case a PCB, has to include an insulating material then contact pressure must depend on the insulating material. The same requirement exists in clause 7.1 f) of IEC 60079-15: 2010. This apparent contradiction has existed for a considerable time and is continuing in the FDIS text for the next edition of 60079-7 for both Ex eb and Ex ec (Clauses 4.2.1 (f) General, 4.2.3.4 Ex eb and 4.2.3.5 Ex ec). Since the reference to edge connectors is in a note, the formal resolution would be to discount the note. However, the note was presumably put there for a purpose, indicating that an edge connector would be acceptable. Question: How, rationally, can the inconsistencies be resolved? Is an edge connector permitted? Draft Answer: Because edge connectors are specifically mentioned as acceptable, the MT of 60079-7 and 60079-15 must have assumed that 4.2.1(f) of 60079-7 and 7.1(f) of 60079-15 did not take precedence in this particular case. The term “edge connector” is also assumed to include pin-header connectors and similar devices. ExCBs are invited to comment, particularly if they have recently issued certification for equipment containing an edge connector. Should there be differentiation between edge connectors used in Equipment EPL GB and EPL Gc? |  |  |