INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE: Draft Decision Sheet – Tensile testing of cable glands

INTRODUCTION

This ExTAG Draft Decision Sheet was prepared as a result of discussions held during the recent IEC TC 31 WG22 meeting.  The document is issued for a 4 week comment period.

Please submit comments using the attached Comments Table by-

Comment to forwarded by 2013 05 10 to

Christine Kane

If no comments are received the document will proceed to publication.

On behalf of Mr. Brenon

- Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body/ Country | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-0:2011 IEC 60079-0:2007 | Clauses: A.3.1.1 A.3.1.4 A.3.1.5 |  |
| --- | --- | --- |
| Subject: Tensile testing of cable glands Status of document: Draft | Key words: - Cable glands - Tensile tests | # Date: Originator of proposal: IEC TC31 WG22 via Chairman ExTAG TC/SC involved: IEC/TC 31 |
| Questions: Is the torque used for the tensile test of A.3.1.4 required to be the same torque as the initial torque determined in A.3.1.1? At what point in the testing sequence is the thermal endurance to heat test conducted? Answer: In A.3.1.1, a torque is applied to either the screws of a flanged compression element or the nut of a screwed compression element to compress the sealing ring to secure the mandrel. A tensile force is then applied to the mandrel to confirm the securement. The tensile force is applied for not less than 6 h. The test is carried out at an ambient temperature of (20 ± 5) °C. The clamping assured by the sealing ring is acceptable if the slippage of the mandrel or cable sample is not more than 6 mm. Subsequently, either the complete cable gland and mandrel assembly tested, or a new sample prepared using the same torque values, is then to be subjected to the thermal endurance tests. The maximum service temperature is considered to be 75 °C unless otherwise specified by the manufacturer. NOTE 1 The 75 °C service temperature is the median of the branching point and entry point temperatures. NOTE 2 Cable glands employing only metallic sealing rings and metallic parts do not require thermal endurance tests. The subsequent test conditions and acceptance criteria are given in A.3.1.4. Prior to the application of the tensile force, the gland may be re-tightened in accordance with the manufacturer’s instructions. NOTE The torque applied to the screws or nut during retightening is not required to be the same as the initial torque applied in A.3.1.1. In A.3.1.5, the “value needed to prevent slipping” is the torque required for A.3.1.4. |  |  |