- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM

FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR

USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Compilation of comments - Draft ExTAG Decision Sheet ExTAG/276/CD – Temperature measurement of cable entries on electrical machines

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document is a compilation of comments, as well as observations, from the originator, IEC TC31 WG27, via the ExTAG Chairman Mr Ron Sinclair, on Draft Decision Sheet ExTAG/276/CD Draft ExTAG Decision Sheet – Temperature measurement of cable entries on electrical machines

A revised document taking into account these comments and observations, ExTAG/276A/CD Draft ExTAG Decision Sheet – Temperature measurement of cable entries on electrical machines, has been circulated for a further one month comment period.

On behalf of Mr. Brenon

- Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body/ Country | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Comment of Chairman ExTAG |
| --- | --- | --- | --- | --- | --- | --- |
| LCIE (FR) |  |  | G | Agreed in principle. It would be clearer to add a conclusion to the answer given | Add the following : “Conclusion : It is acceptable to use the temperature measurement of the internal air space temperature of the terminal box to determine if the temperature under rated conditions is higher than 70 °C at the entry point or 80 °C at the branching point of the conductors” | It is believed that the existing text is sufficiently clear, particularly when the three additional amplification notes are added – see below |
| NANIO CCVE (RU) |  |  | General | We support this draft decision sheet in general, but it is required to clarify the point where the temperature shall be measured. | We propose the following wording: The use of the internal air space temperature at the point of entry of the cable to represent the service temperature of terminal box gaskets and seals, the cable branching point temperature, and the entry point temperature reflects the normal practice of testing machines without prior knowledge of the actual glands and cables to be used for installation. The entry point of the cable where the temperature is measured shall be sealed or hermetically sealed to ensure that there is no air-circulation which can reduce the measured temperature. The production of heat from the machine connections is generally insignificant with respect to the production of heat from the machine windings and core. | The additional text referencing the need to avoid additional cooling from leaving the entry open has been added as additional amplification. |
| SP (SE) |  |  |  | We suggest that the decision sheet applies not only to motors. | The use of the internal air space temperature to represent the service temperature of terminal box gaskets and seals, the cable branching point temperature, and the entry point temperature reflects the normal practice of testing machines equipment without prior knowledge of the actual glands and cables to be used for installation. The production of heat from the machine equipment connections is generally insignificant with respect to the production of heat from the machine windings and core equipment itself. | The text originated from IEC TC31 WG27 specifically in the context of electrical machines. The possibility of applying the same principle to other equipment has been added as additional amplification. |
| UL (US) |  |  | General | UL-USA supports this DS. |  |  |
| UL/DEMKO (DK) |  |  | General | ULD supports this draft DS. |  |  |
| VTT (FI) |  |  | T | The use of the internal air space temperature shall be restricted to represent only the cable branching point temperature, and the entry point temperature. The service temperature of terminal box gaskets and seals shall be measured direct from the enclosure walls, because those are heated by machine frame by heat conduction. | The use of the internal air space temperature to represent the cable branching point temperature, and the entry point temperature reflects the normal practice of testing machines without prior knowledge of the actual glands and cables to be used for installation. The production of heat from the machine connections is generally insignificant with respect to the production of heat from the machine windings and core | Additional amplification has been added to confirm that this method is not intended to apply to a gasket between the frame of the motor and the terminal box |