INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

Title: 	Draft ExTAG Decision Sheet - Repair facility records

Circulation to: ExTAG Members

##### Introduction

This Draft Decision Sheet, prepared by BASEEFA, has been issued for discussion during

the Calgary 2012 ExTAG Technical Meeting.

Michel Brenon

Michel Brenon

ExTAG Secretary

| Address: Standards Australia Building 286 Sussex Street Sydney NSW 2000 Australia | IECEx Secretariat Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 e-mail: chris.agius@iecex.com http://www.iecex.com | Mr Michel Brenon LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 Email: michel.brenon@lcie.fr |
| --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-19:2010 | Clauses: 4.4.1.5.3 |  |
| --- | --- | --- |
| Subject: Repair facility records Status of document: | Key words: Attestation of conformity for replacement components | # Date: 16th July 2012 Originator of proposal: Baseefa TC/SC involved: IEC/TC 31 |
| Question: a. What is acceptable as an ‘attestation of conformity’? Background: IEC 60079-19:2006 called for a certificate of conformity to be included in job records for components used in repaired equipment. A ‘certificate of conformity’ is a recognized document normally issued by the component supplier/manufacturer which certifies conformity with purchase order requirements. IEC 60079-19:2010 has been revised and calls for an ‘attestation of conformity’ in place of the certificate of conformity’. b. Are all replacement components covered by this requirement? Background: While carrying out audits on various repair workshops there is evidence of confusion as to which components are covered by this requirement. One view is that items such as bearings and seals(proprietary items) are not covered by this requirement. Another view is that any item affecting the type of explosion-protection is covered by this requirement. A further view is that fasteners are not covered by this requirement The issue of fasteners is important with regard to bolts holding FLP covers on Exd enclosures and machines. Answer: - Attestation of Conformity should be interpreted as Certificate of Conformity, or a document of equivalent status, in order to make the requirement clear. - The intention of the requirement is to have a certificate of conformity for all components which could affect the type of explosion-protection. As such the clause should be read as: ‘certificate of conformity for all components which affect the type of explosion-protection’. |  |  |