INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE

IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE: Draft Decision Sheet - Draft ExTAG Decision Sheet – What Standards must be listed on page 2 of the IECEx Certificate?

INTRODUCTION

This updated ExTAG Draft Decision Sheet ExTAG/243A/CD ExTAG Draft Decision Sheet – What Standards must be listed on page 2 of the IECEx Certificate?  has been prepared taking into account  comment received, on ExTAG/243/CD.

It is now issued for a 4 week comment period.

Please submit comments using the attached Comments Table by-

Comment to forwarded by 2012 04 23 to

Christine Kane

If no comments are received the document will proceed for publication.

On behalf of Mr. Brenon

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body/ Country | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

IECEx / ExTAG DECISION SHEET

Title: ExTAG Decision Sheet – What Standards must be listed on page 2 of the IECEx Certificate?

| Standard: #### All | Clause: | Sheet No. |
| --- | --- | --- |
| Subject: What Standards must be listed on page 2 of the IECEx Certificate? | Key words: - Certificate - Standards - Revision Numbers - Issue - Marking | # Date: 2011-11-14 Originator of proposal: TestSafe Australia TC/SC involved: Nil |
| Related documents: | Nil |  |

Introduction:

There are some differences in opinion on how to prepare certificates where the equipment (call it Ex d Gate End Box) may be made up of several separately certified Ex equipment or components.

The separately certified Ex equipment or components may have been issued with IECEx certificates to different types of protection and Standards (e.g. Ex m, Ex i, Ex e etc) and the Standards may have different issue numbers as well (one Ex i level transducer may have used IEC 60079-0:2011 with IEC 60079-11:2011, another Ex i level transducer may have been certified some time back and may have used IEC 60079-0:2004 with IEC 60079-11:2006, An Ex m encapsulated coil may have used IEC 60079-0:2007 with IEC 60079-18:2009).

When preparing the report and certificate for the complete equipment (Gate End Box) this is predominantly an Ex d box. , and therefore we would be applying IEC 60079-0:2011, read with IEC 60079-1:2007. Inside the equipment description we would be describing all the Ex equipment and components that make up the complete Gate End Box, with all the Ex certificates that have been used for these separately certified equipment.

Question 1:  When we prepare the certificate for the complete equipment, are we supposed to list on page 2 just the major Standards IEC 60079-0:2011 and IEC 60079-1:2007?

Or are we supposed to list all the 10 or more different Standards that are used by the separately certified equipment that make up the Gate End Box?

Answer 1: All It is sufficient to only list the major Standards applicable, including Edition (or year of issue) shall be listed on the page 2 of the IECEx certificate. Where the various parts of the equipment use different certificates, the IEC Standards on page 2 of the CoC shall list  the ‘oldest’ edition from all the applicable certificates (for example, if Ex d certificate is to IEC 60079-0:2011, but Ex e certificate is to IEC 60079-0:2007, the CoC will list IEC 60079-0:2007), unless a gap analysis has been conducted and reported to determine if the product still meets the requirements of the standards being stated in the IECEx Certificate.

In addition, with complete details of the certificates for the separately certified equipment and components shall be provided in the equipment description.

Question 2: Should the The marking code must list only the major type of protection? If practical, it may include the other types of protection applicable too.

Answer 2: The marking code must list all the major types of protection used by the equipment and its parts, with the symbols for the various types of protection to appear in alphabetical order, as provided in IEC 60079-0:2011 Clause 29.6. If practical, it may include the other types of protection applicable too.

Question 3: Should the The certificate only must list the IECEx TR and QAR for the major type of protection for the overall equipment?

Answer 3: The certificate must list all the IECEx TR’s and QAR’s relevant to the separately certified equipment or components.