INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

Title: 	Compilation of comments on ExTAG/215/CD Draft revised ExTAG Decision Sheet – ExTAG 2010/007- Sealing test of cable glands

Circulation to: ExTAG Members

##### Introduction

This document is a compilation of comments on ExTAG/215/CD Draft revised ExTAG Decision Sheet –ExTAG 2010/007- Sealing test of cable glands

This document is issued for consideration during the 2011 Split ExTAG Meeting.

Michel Brenon

Michel Brenon

ExTAG Secretary

| Address: Standards Australia Building 286 Sussex Street Sydney NSW 2000 Australia | IECEx Secretariat Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 e-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body/ Country | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |
| MSTC AU |  |  |  | MSTC supports the proposed amended document ExTAG/215/CD |  |  |
| NANIO CCVE RU |  |  | General | We support ExTAG/215/ CD Draft revised ExTAG Decision Sheet – ExTAG 2010/007- Sealing test of cable glands. We have one remark. Add the following remark to the answer 1) “Yes” relating to IEC 60079-0:2007: Yes, if the obligatory requirements of retightening during the process of use are specified |  |  |
| SP | 1) in red | Answer | te/ed | Re. the answer to question 1) relating to IEC 60079-0:2007, we suggest to clarify the requirements on the manufacturer instructions. | Amend the answer to read: “Yes, when the manufacturer's instructions state the gland is to be checked for tightness within a specified time after installation and at suitable intervals, possibly in accordance with EN 60079-14.” |  |
| UL- USA | 60079-0 Annex A |  | technical | UL does not support the answer to the new question 1: “Is it permitted to retighten a cable gland in accordance with the manufacturer’s instructions prior to carrying out sealing and mechanical testing following hot and cold thermal endurance testing?” When a steel mandrel is used for mechanical tests, the effects of cable degradation have already been accounted for. Any slippage or loosening of the mandrel in this case is due to the effects of aging of the gland itself. Therefore, retightening after thermal endurance testing is not appropriate for mechanical tests. This aligns with ATEX clarification sheet ExNB/09/345/CS. UL supports the remainder of the proposed DS (new questions #2 and #3). | Change the answer to the new question #1 from Yes to No. |  |
| UL/DEMKO DK |  |  |  | Please consider UL USA's comments as UL Demko's comments too. |  |  |
| ZA |  |  |  | The South African National Committee find the decision to allow retightening the gland after thermal cycle test not fair for our local conditions. It is very common to have quite a high delta temperature between day and night followed by heavy rains. After there will less monitoring of the outcome of the glands. Retightening prior to sealing test doesn't prove anything. On the contrary the retightening should be prohibited. That should take us closer to what is happening in the field. |  |  |