INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE

IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group.

TITLE: Draft ExTAG Decision - Determining compliance with winding requirements for enameling.

INTRODUCTION

This proposed Draft ExTAG Decision Sheet was prepared by Underwriters Laboratories Inc, US, and is issued for your consideration.

Please submit comments by -

23rd December 2010 to the ExTAG Secretary, Mr. Michel Brenon, via

Christine Kane

If comments have not been received by the above date this document will be published as an ExTAG Decision Sheet.

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-7 | Clauses: - - 4th Edition 4.7.1.2 – 3rd Edition |  |
| --- | --- | --- |
| Subject: Determining compliance with winding requirements for enameling Status of document: Draft | Key words: - Winding - Wire - Enamel - Grade | # Date: 2010-11-09 Originator of proposal: UL TC/SC involved: IEC/TC31 |
| Question: If a wire manufacturer’s specification is provided that specifies the enamel grade of a winding in accordance with the applicable IEC standard (i.e. IEC 60317-3, IEC 60317-7, IEC 60317-8, or IEC 60317-13), is it required that the ExTL and ExCB verify conformity of the winding to the manufacturer’s specification regarding the enamel grade? Answer: No, it is not a requirement of this standard that conformity to the manufacturer’s specification regarding the enamel grade of the winding needs to be verified. Conformity can be determined based on the manufacturer’s specification. |  |  |