INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulation to: ExTAG – IECEx Testing and Assessment Group

Title: Draft OD 010 - Guidance for the Development, Compilation, Issuance and Receipt of New or Revised IECEx Test Report (ExTR) Documents by IECEx Testing Laboratories (ExTLs) and IECEx Certification Bodies (ExCBs)

INTRODUCTION

Document ExTAG/192/CD - IECEx Draft Operational Document OD 010 Edition 2 - Guidance for Developing New or Revised IECEx Test Report (ExTR) Documents and for Issuing and Receiving Existing IECEx Test Report (ExTR) Documents was circulated to the ExTAG for comment in early 2010. Comments on this document, along with observations received from the convenor, have been circulated in ExTAG/208A/CC Compilation of comments on ExTAG/192/CD IECEx Draft Operational Document OD 010 Edition 2 - Guidance for Developing New or Revised IECEx Test Report (ExTR) Documents and for Issuing and Receiving Existing IECEx Test Report (ExTR) Documents.

This document, a revised edition of ExTAG/192/CD, has been prepared taking into account comments received and is issued for discussion during the ExTAG Berlin Meeting.

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

CONTENTS

1	Introduction	3

2	Requesting and Assigning New or Revised ExTR Documents	3

2.1	Requesting New or Revised ExTR Documents	3

2.2	Assigning New or Revised ExTR Documents	4

3	Developing and Publishing New or Revised ExTR documents	4

3.1	Developing New or Revised ExTR documents	4

3.2	Publishing New or Revised ExTR Documents	9

4	Compiling, Approving and Issuing ExTR packages	9

4.1	General Rules	9

4.2	Ex Test Report	10

4.3	ExTR Cover	11

4.4	ExTR Addendum	13

4.5	ExTR of National Differences	14

5	Receiving, Reviewing and Auditing ExTR Packages	14

Annex A – Helpful Hints for ExTR Developers	16

Annex B – Examples of Completed ExTR Documents	18

# Introduction

The purpose of this Operational Document is to establish a common approach amongst all IECEx Test Laboratories (ExTLs) and IECEx Certification Bodies (ExCBs) operating within the IECEx System regarding the following:

- Requesting and Assigning new or revised ExTR documents.

- Developing and Publishing new or revised ExTR documents.

- Compiling, Approving and Issuing ExTR packages.

- Receiving, Reviewing and Auditing ExTR packages.

The term “ExTR document” refers to any of the IECEx Test Report (ExTR) documents that include the Cover, Addendum and National Differences documents, along with the individual Test Reports for each of the IEC Ex standards.

The term “ExTR package” refers to the overall record of the evaluation and testing performed on a given item or product. An ExTR package is comprised of an ExTR Cover and one or more associated ExTR documents.

Regarding the development of ExTR documents, the term “new” ExTR document refers to the need for the development and publication of the first ExTR document for a given IEC Ex standard, regardless of the edition of the IEC Ex standard.

The term “revised” ExTR document refers to the subsequent development and publication of an ExTR document for a given IEC Ex standard after the first development and publication. The need for a “revised” ExTR document can be due to a new edition of the IEC Ex standard, or due to a change needed to an existing ExTR document to better address the same edition of the IEC Ex standard.

Operational Document (OD) 010 complements the IECEx Operations Manual, OD 009. Terminology has been aligned to comply with IECEx 02, 3rd edition.

The 2nd edition of this OD is issued as a result of joint discussion of ExTAGWG01 and ExTAGWG05 during the 2009 Melbourne IECEx meetings, and as a result of NC comment from the 2010 Australia IECEx meetings. Substantial changes made since the previous edition impact all aspects of the document.

# Requesting and Assigning New or Revised ExTR Documents

## Requesting New or Revised ExTR Documents

### Initial request

The request for a new or revised ExTR document is made by an ExTL/ExCB that has identified an evaluation and testing need under a specific IEC Ex standard that is not suitable addressed by any currently published ExTR documents.

Such requests shall be addressed to the Convenor of ExTAGWG01 in writing at the following email address: Paul.T.Kelly@us.ul.com.

### Request confirmation

Upon receipt of an initial request, the ExTAGWG01 Convenor confirms whether the requested new or revised ExTR is:

- Already developed and available,

- In process of being developed by an already assigned ExTL/ExCB, or

- Not developed and not in process of being developed.

If a requested new or revised ExTR document is already developed and available, this information is conveyed to the requesting ExTL/ExCB.

If a requested new or revised ExTR document is in process of being developed, this information is conveyed to the requesting ExTL/ExCB with an indication of when the new or revised ExTR document will be made available on the IECEx system website.

## Assigning New or Revised ExTR Documents

If a requested new or revised ExTR document is not developed and not in process of being developed, the ExTAGWG01 Convenor will formally assign the request to an ExTL/ExCB as follows:

New ExTR documents

- The requesting ExTL/ExCB will be asked if it is prepared to develop the new ExTR document within a determined timeframe.

- If the requesting ExTL/ExCB is not prepared to develop the new ExTR, the ExTAGWG01 Convenor immediately starts a formal consultation among other member ExTLs/ExCBs to seek a body willing to develop the new ExTR.

- Should the consultation made by the ExTAGWG01 Convenor among other member ExTLs/ExCBs lead to a no-offer, the requesting ExTL/ExCB will either need to develop the new ExTR, or the request will not proceed.

Revised ExTR documents

- The ExTL/ExCB that developed the original ExTR document will be asked if they are prepared to develop the revised ExTR document within a determined timeframe.

- If the ExTL/ExCB that developed the original ExTR document is not prepared to develop the revised ExTR, the ExTAGWG01 Convenor will ask the requesting ExTL/ExCB if it is prepared to develop the new ExTR document within a determined timeframe.

- If the requesting ExTL/ExCB is not prepared to develop the new ExTR, the ExTAGWG01 Convenor immediately starts a formal consultation among other member ExTLs/ExCBs to seek a body willing to develop the new ExTR.

- Should the consultation made by the ExTAGWG01 Convenor among other member ExTLs/ExCBs lead to a no-offer, the requesting ExTL/ExCB will either need to develop the revised ExTR, or the request will not proceed.

# Developing and Publishing New or Revised ExTR documents

## Developing New or Revised ExTR documents

### General rules and formatting requirements

The following guidelines should be taken into account when developing new or revised ExTR documents:

File format	MS Word format, .doc

Font	ExTR main heading: Normal text, Arial 12 pt

All other headings and text: Normal text, Arial 10 pt

Paper size	A4 (210 x 297 mm)

Margins	1.3 cm (0.53 in) Top; 2.2 cm (0.86 in) Bottom; Left 2.2 cm (0.88 in); Right 2.2 cm (0.88 in), 0.0 cm (0.0 in) Gutter, 1.3 cm (0.5 in) Header, 1.3 cm (0.5 in) Footer

Language	English (US)

Header	“ExTR Reference No.” is the only header text, and shall be located in the upper-right corner. This header text is to appear on all pages of the involved ExTR document.

Footer	“TRF No.” and “Page X of Y” are the only footer text, with the “TRF No.” located in the lower-left corner and “Page X of Y” located in the lower-center. Both footer text are to appear on all pages of the involved ExTR document.

TRF Number	The naming convention for Test Report Form (TRF) numbers is as follows:

      - “ExTR”; followed by

      - the IEC standard number (without the “IEC” prefix); followed by

      - an underscore; followed by

      - an Arabic number reflecting the edition of the IEC standard; followed by

      - an uppercase letter A indicating the version of the ExTR document.

The example below would be the ExTR identification for the initial version of an ExTR blank document for IEC 60079-6, 3rd edition.

ExTR60079-6_3A

Subsequent revisions to this ExTR blank, not due to an edition change in the standard, would replace the “A” with sequentially increasing letters in the alphabet.

### ExTR structure

An ExTR document must be easy to fill in and should include no macros, except for the ExTR Reference Number field, the Page X of Y field, and the Date of issue field. No protected fields should be included. See Annex A for instructions on various Word features to facilitate document development.

While an ExTR package is comprised of an ExTR Cover and one or more associated ExTR documents, its overall structure can be divided into four main sections: 1) the ExTR Cover, 2) the ExTR Body, 3) the Measurement Section and 4) the ExTR of National Differences.

The ExTL should always compile and review the first three sections for approval by the ExCB. If National Differences are included, then the appropriate fourth section shall also be compiled, reviewed and approved.

### ExTR Cover

An ExTR Cover is the sole top-level document to associate together all other parts of an ExTR package.

The role of the ExTR Cover for an ExTR package is to contain key information that is common across all other parts of the ExTR package. This eliminates the need to repeat such common information across multiple ExTR documents, and highlights this key information in one location for ready access.

Each ExTR Cover includes:

- Names and addresses of the ExTL, ExCB, Applicant and Manufacturer;

- Names and signatures of the responsible ExTL and ExCB staff;

- Standard(s) used for the evaluation;

- Details on where and how testing was performed or witnessed;

- Whether any National differences were considered;

- Product name and rating(s);

- Specific conditions of use, if any; and

- Routine tests, if any.

An example of a completed ExTR Cover will be included as an Annex B-1 as part of the next update to this operational document.

### ExTR Body

This part of the ExTR package is a standardized section consisting of one or more ExTR checklist tables that make reference, clause by clause, to the requirements of a particular Ex standard. This is applicable to Ex Test Reports, ExTR Addendums and ExTRs of National Differences.

Note: Sub-clauses in Ex standards that are explanatory notes do not need to be detailed in the checklist.

The checklist table portion of an Ex Test Report, ExTR Addendum or ExTR of National Differences is referred to as the “test case” section. Each “test case” section is formatted into four columns of an overall single table as follows: 1) Clause, 2) Requirement – Test, 3) Result – Remark and 4) Verdict.

| Clause | Requirement – Test | Result – Remark | Verdict |
| --- | --- | --- | --- |

As the table extends from one page to the next, the table heading should repeat at the beginning of each new page. If this is not practical for some reason, the table may be split into several separate tables, as long as the headers are repeated on each page.

In order to best utilize the available space on each page in a balanced fashion, the width of each column in the “test case” section should be as follows: 2.16 cm (0.85 in), 5.08 cm (2.00 in), 7.75 cm (3.05 in) and 2.16 cm (0.85 in).

### Clause

### The “Clause” column is for a reference to each Ex standard clause or sub-clause.

### Requirement – Test

The “Requirement – Test” column is for the text of the heading, or for a summary of the requirement or test detailed in the related clause or sub-clause.

Any summary wording used in the “Requirement – Test” column needs to be simple, and needs to prompt for a clear “Pass” or “N/A” verdict. The full text of a paragraph from the Ex standard is not to be repeated.

Different “test case” types may be presented in one of the following ways (each type is followed by an example):

- Main Clause Heading: No “Verdict” is rendered since no requirements are associated with the clause in the Ex standard.

| Clause | Requirement – Test | Result – Remark | Verdict |
| --- | --- | --- | --- |
| 5 | Flameproof joints |  |  |

- Main Clause Heading: “Verdict” is rendered since requirements are associated with the clause in the Ex standard.

| Clause | Requirement – Test | Result – Remark | Verdict |
| --- | --- | --- | --- |
| 9 | Light-transmitting parts |  |  |

- Sub-Clause Heading: No “Verdict” is rendered since no requirements are associated with the sub-clause in the Ex standard.

| Clause | Requirement – Test | Result – Remark | Verdict |
| --- | --- | --- | --- |
| 5.2 | Non-threaded joints |  |  |

- Sub-Clause Heading: “Verdict” is rendered since requirements are associated with the sub-clause in the Ex standard.

| Clause | Requirement – Test | Result – Remark | Verdict |
| --- | --- | --- | --- |
| 6.2 | Mechanical strength of equipment |  |  |

Clauses or Sub-clauses involving requirements, b) and d) above, are referred to as regular “test case” clauses. Regular “test case” clauses can benefit from explanatory details under the “Result – Remark” heading which support the Verdict being entered.

Some regular “test case” clauses make it mandatory that specific details be entered under the “Result – Remark” heading in support of the “Verdict”. A dotted leader as shown in the example below identifies such instances.

| Clause | Requirement – Test | Result – Remark | Verdict |
| --- | --- | --- | --- |
| 5.2.1 | Width of joints (L)....................: |  |  |

### Result – Remark

The “Result – Remark” column is for explanatory details that result from the related “Requirement – Test”, and that support the ultimate “Verdict”. Examples of such details include a measured value, a test result or an explanatory remark.

To best facilitate subsequent acceptance of an ExTR package by a Receiving Body, most regular “test case” clauses can benefit from such explanatory details under the “Result – Remark” heading.

- Verdict

The “Verdict” column is for stating the judgment resulting from analysis based on the “Requirements – Test” column, and reflective of the explanatory details under the “Result – Remark” column.

Allowed verdict choices are:

- Pass

- N/A (Not applicable)

An example of a completed page from an ExTR Body will be included as an Annex B-2 as part of the next update to this operational document.

### Measurement Section

When completing the checklist portion of the ExTR Body, there is often a need to detail measured values and test results that do not easily fit in the fields under the “Result – Remark” column.

In such situations, an area for additional narrative remarks and tables of measured values, specific test conditions and test results can and should be provided for in this Measurement Section of the ExTR package.

An example of a completed page from a Measurement section will be included as an Annex B-3 as part of the next update to this operational document.

### ExTR of National Differences

An ExTR of National Differences is to supplement an Ex Test Report or ExTR Addendum, with a separate such document issued for each intended country / region.

The structure of an ExTR of National Differences is similar to the checklist and “test case” section structure of the ExTR Body described above.

An example of a completed page from an ExTR of National Differences will be included as an Annex B-4 as part of the next update to this operational document.

### Special instructions for ExTR Developers

Developers of ExTR documents shall use the agreed ExTR format and templates detailed in this OD. The only “free-form” section open for Developer-specific variance is the Measurement Section.

The Measurement Section of an ExTR document may be tailored as deemed necessary by the ExTR Developer to address the specifics of the involved IEC Ex standard and the products commonly covered by the standard.

Issues such as selection of test items, particulars or layout of tables can be addressed as needed. Tables of larger size in the Measurement Section can be displayed and printed in “landscape” orientation.

## Publishing New or Revised ExTR Documents

When the assigned ExTL/ExCB completes development of the new or revised ExTR document, the resulting ExTR document is forwarded to the ExTAGWG01 Convenor in Word format via email.

Upon receipt of a newly developed or revised ExTR document, the ExTAGWG01 Convenor will verify the Test Report Reference Number, layout, editing, etc. is valid prior to submitting the ExTR document to the IECEx Secretariat for distribution.

The IECEx Secretariat informs all member ExCBs and ExTLs through emails each time a new or revised ExTR document is published and posted on the IECEx System website.

# Compiling, Approving and Issuing ExTR packages

The content for all blank ExTR documents (i.e. the form text and layout) are under copyright ownership © 2010 by the International Electrotechnical Commission System for Certification to Standards Relating to Equipment for use in Explosive Atmospheres (IECEx System), Geneva, Switzerland. All rights reserved.

This form text and layout of blank ExTR documents may be reproduced in whole or in part for non-commercial purposes as long as the IECEx System is acknowledged as copyright owner and source of the material. The IECEx system takes no responsibility for, and will not assume liability for, damages resulting from the reader's interpretation of the reproduced material due to its placement and context.

The following guidance shall be taken into account when compiling, approving and issuing an ExTR package.

## General Rules

While an ExTR package is comprised of an ExTR Cover and one or more associated ExTR documents, its overall structure can be divided into four main sections: 1) the ExTR Cover, 2) the ExTR Body, 3) the Measurement section and 4) the ExTR of National Differences.

The ExTL should always compile and review the first three sections for approval by the ExCB. If National Differences are included, then the appropriate fourth section shall also be compiled, reviewed and approved.

The test results presented in an ExTR package relate only to the item or product tested.

For entry of data or additional details beyond the standard fields of an ExTR package, the following applies:

      - "(see Attachment #)" refers to additional information appended to the ExTR package.

      - "(see appended table)" refers to a table appended to the ExTR package.

      - Throughout this ExTR package, a point is used as the decimal separator.

As a separate issue from the form text and layout of blank ExTR documents, the technical content of a completed ExTR package shall not be reproduced except in full without the written approval of both the Issuing ExCB and ExTL.

## Ex Test Report

### General

An Ex Test Report provides a clause-by-clause documentation of the initial evaluation and testing that verified compliance of an item or product with an IEC Ex standard.

Note: Supplemental evaluation and testing to address alternate constructions or modifications are documented as part of an ExTR Addendum. See ExTR Addendum details below.

Such Ex Test Reports are part of ExTR packages that may include other Ex Test Report, Addendum and National Differences documents, along with a single ExTR Cover.

The Ex Test Report portion of an ExTR package is a standardized section consisting of one or more checklist tables that make reference, clause by clause, to the requirements of a particular IEC Ex standard.

An Ex Test Report is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the Ex Test Report as part of the overall ExTR package on the associated ExTR Cover.

Regarding the four main sections that comprise the overall structure of an ExTR package, the Ex Test Report contains two of those four parts: 1) the ExTR Body, and 2) the Measurement Section.

### ExTR Body

The first two columns in the ExTR Body checklist titled: “Clause” and “Requirement - Test” are provided for reference only and should not be modified by the ExTL. For the complete text of the requirement, the ExTL should reference the involved IEC Ex standard.

The next column in the ExTR Body checklist titled “Result - Remark” is to be completed as part of the investigation. In this column a measured value or an explanation shall be entered to detail the results from the “Requirement – Test” column.

The “Result - Remark” column in the ExTR Body confirms the compliance or declares the particular clause as not applicable (the verdicts can be either “Pass” or “N/A”). There are different “test case” types, and the ExTL should pay attention to the type of the clause in order to correctly complete the ExTR package.

When entering “Result – Remark” field comments, such comments shall clearly indicate why the particular item passed or is not applicable.

To facilitate acceptance by Receiving Bodies without re-testing, ExTR documents shall provide sound, sufficient information supporting the Issuing Body’s evaluation and testing. To further facilitate such acceptance, ExTR documents should include any additional information that will help the Receiving Body. Examples of such additional information include photos, complete data sheet packages, schematics, installation documents and detailed component information.

If a clause in the ExTR Body checklist involves a number of sub-clauses, and nothing under the clause or it’s related sub-clauses are applicable to the involved evaluation and testing, the following actions are to be taken so as to simplify the completed ExTR document:

- In the “Result – Remark” field for the main clause, a comment is to be entered indicating the reason for the entire clause not being applicable;

- In the “Verdict” field for the main clause, “N/A” is to be entered; and

- All the rows for the related sub-clauses to the main clause are to be deleted.

### Measurement Section

This part of the ExTR Body contains tables, and may contain areas for special remarks that can provide a narrative of the evaluation and testing that determined compliance of the ietm or product.

Tables are provided to record actual measurements made during the evaluation and testing. Tables also provide room for information about test conditions.

## ExTR Cover

### General

An ExTR Cover is the sole top-level document to associate together all other parts of an IECEx Test Report (ExTR) package.

An overall ExTR package is comprised of an ExTR Cover and one or more associated ExTR documents (which may include Ex Test Reports, ExTR Addendums and ExTR of National Differences).

All ExTR package documents are compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the overall ExTR package on this ExTR Cover.

### Standards field

The Standards field of the ExTR Cover is for identifying the standard number and edition or year of issue for all standards associated with the ExTR package.

### Rated service temperature range field

The Rated service temperature range field of the ExTR Cover is for indicating the rated service temperature range when the ExTR package addresses an Ex Component.

### Testing performed or witnessed at a remote location field

Two separate fields are provided as part of the ExTR Cover for including details regarding who performed or witnessed all the involved testing, and where this testing occurred.

The first field addressing this topic appears on the first page of the ExTR Cover.  It is a ‘Yes/No’ field to simply highlight which one of the following scenarios apply:

- Was all testing fully* performed by ExTL staff at the ExTL address noted on the ExTR Cover?; or

- Was all testing fully* performed or witnessed by ExTL staff, with some or all of the testing involving a location other than the ExTL address noted on the ExTR Cover?

*Note: For any long term testing to be considered “fully” performed or witnessed, ExTL staff shall be present at the start and conclusion, with continuous monitoring of conditions in between.

The second field addressing this topic appears further into the ExTR Cover.  It is a free-form text field to detail specific tests and remote locations for any remote testing, along with an indication that the remote location was determined by the ExTL to have the environment and the capabilities necessary to perform the involved testing.

If any electronic medium such as video was used for “live” witnessing of any testing, this shall also be detailed.

If all testing was performed at the ExTL address noted on the ExTR Cover, ‘N/A’ is all that needs to be entered in this second field.

### National differences considered field

Two separate fields are provided as part of the ExTR Cover for including general details regarding whether or not any National differences were considered, and if they were considered, how this consideration was addressed.

The first field addressing this topic appears on the first page of the ExTR Cover.  It is a ‘Yes/No’ field to simply indicate if National Differences were considered as part of the ExTR package and, if they were, if this consideration involved the issuance of an ExTR of National Differences.

The second field addressing this topic appears further into the ExTR Cover.  It is a free-form text field to indicate, in general, why an ExTR of National Differences was or was not issued if National Differences were considered.

The following is an example of text that could be entered in the second field to explain why the EN National Differences to IEC 60079-0:2004, IEC 60079-11:2006 and IEC 60079-26:2006 were considered as part of the evaluation and testing of an item or product, but no ExTR of National Differences were issued:

“The change to the product involving the optional housing was reviewed with respect to the National Differences for EN 60079-0:2006, EN 60079-11:2007 and EN 60079-26:2007 and found to be in compliance with these requirements.

No ExTRs of National Differences were issued because:

- There are no National Differences between IEC 60079-11:2006 and EN60079-11: 2007.

- The optional housing does not affect any National Differences between IEC 60079-0:2004 and EN 60079-0:2006.

- The optional housing does not affect any National Differences between IEC 60079-26:2006 and EN 60079-26:2007.”

If no National Differences were considered, ‘N/A’ is all that needs to be entered in this second field.

Neither of these fields are for the actual recording of any specific “Result – Remark” details regarding any specific National Differences. The ExTR of National Differences is the document for recording any specific “Result – Remark” details.

### Copy of Marking Plate field

The Copy of Marking Plate field of the ExTR Cover is for including a copy of the item or product Marking Plate. A copy need not be included if the ExTR package is not supporting an IECEx CoC.

For instances in which a copy of the Marking Plate is not included, a statement is to be entered indicating the reason for the exclusion.

### General product information field

The General product information field of the ExTR Cover is for including any nomenclature details, such as a description of the type designations, if applicable.

### Specific Conditions of Use field

The Specific Conditions of Use field of the ExTR Cover is for detailing any required Specific Conditions of Use from any of the associated Ex Test Report, Addendum or National Differences documents that comprise the ExTR package. If there are no required Specific Conditions of Use, then “N/A” is to be entered in this field.

### Routine tests field

The Routine tests field of the ExTR Cover is for detailing any required Routine tests from any of the associated Ex Test Report, Addendum or National Differences documents that comprise the ExTR package.  If there are no required Routine tests, then “N/A” is to be entered in this field.

## ExTR Addendum

An ExTR Addendum is only issued to supplement a previously issued Ex Test Report (which may include other previously issued ExTR Addendums).

Supplemental modifications that could require an ExTR Addendum include:

      - Design changes impacting the method of protection, such as…

- Changes to the electrical rating

- New or alternate components added

- Enclosure material / thickness changes

- Circuit layout changes

      - Addition of model/type numbers as part of a product series

      - Name/address changes regarding Applicant/Manufacturer

      - Corrections to the original test report documentation

Similar to the function of an Ex Test Report, an ExTR Addendum provides a clause-by-clause documentation of the evaluation and testing that verified compliance of an item or product with an IEC Ex standard. The difference between the two is that the Ex Test Report provides documentation of the initial evaluation and testing of an item or product, while an ExTR Addendum provides documentation of a supplemental modification to an item or product.

Note: See Ex Test Report details above for related guidance when compiling, approving and issuing an ExTR Addendum.

The test results presented in an ExTR Addendum are only valid when considered together with the related Ex Test Report that was previously issued (along with any other previously issued ExTR Addendums for the same item or product).

Only those clauses applicable to the supplemental issue being addressed are to be tabulated and remarked upon as part of an ExTR Addendum. An ExTR of National Differences may also supplement this document.

When an item or product is subject to supplemental modification, the ExTR Cover under “General product information” shall clearly identify the nature of such modifications and corrections, as applicable.

When a supplemental modification is submitted to a previously issued Ex Test Report, there are two approaches to addressing the modification. One approach is to issue an ExTR Addendum. The other approach is to issue a new Ex Test Report that incorporates the previously issued Ex Test Report, together with any previously issued ExTR Addendums and the latest supplemental modification.

While it is at the discretion of the Issuing Body as to which approach to follow for any supplemental modification submittal, it is recommended that no more than three modifications be allowed to a previously issued Ex Test Report. After the third modification, a new Ex Test Report should be issued that incorporates the previously issued Ex Test Report, together with any previously issued ExTR Addendums and the latest supplemental modification.

The rationale behind limiting the number of modifications to a previously issued Ex Test Report is to avoid confusion regarding what is actually covered by the combination of the Ex Test Report and any subsequent ExTR Addendums.

This is not intended to preclude issuing a new ExTR package on a more frequent basis, if the ExCB deems it appropriate. It is a recommended practice to consider issuing a new ExTR package anytime an extensive supplemental modification is being submitted.

An ExTR Addendum is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the ExTR Addendum as part of the overall ExTR package on the associated ExTR Cover.

## ExTR of National Differences

An ExTR of National Differences is to supplement an Ex Test Report or ExTR Addendum, with a separate such document issued for each intended country / region.

All National Differences of the intended country / region are to be tabulated and remarked upon as part of this document, even those that are not applicable.

Similar to the function of an Ex Test Report and ExTR Addendum, an ExTR of National Differences provides a clause-by-clause documentation of the evaluation and testing that verified compliance of an item or product with an Ex standard. However, while an Ex Test Report and Ex Addendum document compliance with an IEC Ex standard, an ExTR of National Differences documents compliance with country / regional Ex standards.

Note: See Ex Test Report details above for related guidance when compiling, approving and issuing an ExTR of National Differences.

An ExTR of National Differences is to be compiled and reviewed by the ExTL. The Issuing ExCB indicates final approval of the ExTR of National Differences as part of the overall ExTR package on the associated ExTR Cover.

# Receiving, Reviewing and Auditing ExTR Packages

The effective exchange of ExTR packages between Issuing and Receiving ExCBs is essential to the ability of manufacturers to obtain multi-country / multi-region certifications based on a single test program and single submission of samples.

To accomplish this IECEx System objective, ExTR documents need to be developed and maintained in a timely and user-friendly fashion, and ExTR packages need to be compiled and approved by the Issuing Body with enough detail to clearly support all technical decisions.

If these actions are done effectively (from development through issuance), such an approach can help facilitate the ability of a Receiving Body to review and audit an ExTR package without requiring re-testing or re-submittal of samples.

However, In accordance with IECEx 02, a Receiving ExCB may request a sample of the Ex equipment and copies of the documentation referred to in an ExTR Cover if such is deemed necessary.

# Annex A – Helpful Hints for ExTR Developers

1. In order to add automatic update of all fields containing the same information, for example the ExTR Reference No., follow these instructions:

Select View, Toolbars, Forms

Move cursor into the space for ExTR Reference No. on the first page of the Test Report.

Then click on Text Form Field icon (ab). Select shading for easier identification of this new field.

Double click on the newly created Text Form Field to open the Text Form Field screen:

Type Bookmark name CP1 (use name CP1 for the ExTR Reference No. for consistency).

Type Report Ref. No. in the “default text field” and click OK.

Move cursor to any space where normally the ExTR Reference No. should be typed (normally in the header).

Select Insert, Field, Links and References and Ref. Then type CP1 and click OK.

If you have several sections with different headers and page layout in the TRF you may need

to repeat these steps for each section.

To use the update field feature go to the Text Form Field. Double click on the field, type the

information (such as Report Ref. No.) in the “default text field” and click OK.

On each page of the printed document MS Word will automatically enter the text from the

“default text field” into the selected locations with the REF CP1 identification.

2. In order to add a dotted leader for test cases requiring a specific result or comment in

addition to the verdict, follow these instructions:

Cl.

Requirement - Test

Result

Verdict

20.5 Length ≥10 m ......................................... : 12,5 m Pass

Move cursor to the second cell (“Requirement – Test”) and click Format then select Tabs.

In the “Tab stop position” type 6.7 cm (or as appropriate for the size of the table).

Set the alignment on “left”. Select the leader No. 2. Click on “Set” and then OK.

While still in the second cell, move cursor to the end of text in this cell and press “Control” +

“Tab”. Add the “:” symbol.

3. In order to add shading in the cell, place cursor in that cell; select Format, Borders and

Shading. Click on the “Shading” tab and choose the top square for the 12.5% Grey.

You may also select Patterns, Style and 12.5% for the shading. This method appears to

work better during the conversion from MS Word 6.0 to higher versions of MS Word (the

shading is not lost).

# Annex B – Examples of Completed ExTR Documents

Reserved for Future Use