INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE

IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

TITLE: Revised Draft decision Sheet - Voltage values in determination of creepage and clearance distances

INTRODUCTION

Following on from comments received on ExTAG/181/CD, circulated in ExTAG/184/CC, a revised draft ExTAG Decision Sheet ExTAG/181A/CD has been prepared for your consideration.

Members are advised that following on from the publication of the ExTAG Decision Sheet IEC TC 31 will be requested to issue an Interpretation Sheet (ISH) and that once this has been published this ExTAG Decision Sheet is to be withdrawn.  Both the Decision Sheet and the collation of comments will be supplied to MT 60079-7

Please submit comments by -

29th January 2010 to ExTAG Secretary, Mr. Michel Brenon, via

Christine Kane

If comments have not been received by the above date this document will be published as an ExTAG Decision Sheet.

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-7:2001 IEC 60079-7:2006 | Clause: Table 1 |  |
| --- | --- | --- |
| Subject: Voltage values in determination of creepage and clearance distances Status of document: | Key words: - Creepage and Clearance Distances - Working Voltage | # Decision of members: # Date: Originator of proposal: J. Dinneen/UL TC/SC involved: |
| Question: When determining the creepage and clearance distances from the voltage in Table 1, if the working voltage is between two voltage values described in the table, would the next higher voltage be applied? Background: In IEC 60079-7:1990, and other IEC standards (e.g. IEC 60079-15) it is clear that the user of the standard shall apply the next highest voltage level based on the less than/equal to symbol (<) next to the voltage values. In IEC 60079-7: 2001 and 2006, the less than/equal to symbols are not included in Table 1, so it is not clear whether the next higher voltage shall be applied. This could imply that interpolation between the listed values is permitted. For example: Voltage Minimum Clearance required by 60079-7 - 4,0 250 5,0 If the voltage in question is, for example, 225 V, is the required spacing 5,0 mm, or can it be extrapolated to 4,5 mm? Answer: If the working voltage is between two voltage values described in Table 1, the value for the higher voltage level shall be applied when determining the creepage and clearance distances However, the notes to Table 1 allow the use of the lower voltage value if the working voltage does not exceed the table voltage by more than 10%. The answer to the example question is that the minimum clearance required at 225 V is 5,0 mm. However, for a working voltage of 220V (i.e. 200 + 10%) the minimum clearance required is 4,0 mm. |  |  |