INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Circulated to: ExTAG – IECEx Testing and Assessment Group

- TITLE: Draft Decision Sheet - Sealing test of cable glands - IEC 60079-1 6th 	Edition 2007

INTRODUCTION

This Draft Decision Sheet, prepared by SIRA, is issued for your consideration.

Please submit comments by -

2009 07 08 to ExTAG Secretary, Mr. Michel Brenon, via

Christine Kane

If comments have not been received by the above date this document will be published as an ExTAG Decision Sheet.

## Michel Brenon

ExTAG Secretary

| Address: IECEx Secretariat 286 Sussex Street Sydney NSW 2000 Australia | Contact Details: Tel: +61 2 8206 6940 Fax: +61 2 8206 6272 E-mail: chris.agius@iecex.com http://www.iecex.com | ExTAG Secretary Mr Michel Brenon/LCIE Tel: +33 1 40 95 5519 Fax: +33 1 40 95 5520 e-mail michel.brenon@lcie.fr |
| --- | --- | --- |

| Member Body | Clause/ Subclause | Paragraph Figure/ Table | Type of comment General/technical/ editorial | COMMENTS | Proposed change | Observation |
| --- | --- | --- | --- | --- | --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-1 Sixth Edition 2007 | Clauses: C.3.1 |  |
| --- | --- | --- |
| Subject: Sealing test of cable glands Status of document: Draft | Key words: - Cable glands - Sealing test - Thermal endurance | # Date: 090520 Originator of proposal: D R Stubbings/Sira TC/SC involved: |
| Introduction Clause C.3.1 of IEC 60079-1:2007 requests that cable glands be assembled on to either mandrels or cable (as applicable to their design) in accordance with the manufacturer’s instructions and then subjected to the appropriate thermal conditioning parameters as applicable in accordance with IEC 60079-0. The sealing test is conducted after thermal endurance. Question: Is it permissible to retighten a cable gland in accordance with the manufacturer’s instructions prior to carrying out the sealing test? Answer: It is permitted to tighten the gland prior to the overpressure test when the manufacturer's instructions state the gland is to be checked for tightness within a specified time after installation and at suitable intervals, in accordance with IEC 60079-17. In addition it is important that that the manufacturer’s instructions also state that it is the seal compression component that is to be checked and tightened as necessary (IEC 60079-17:2007 cl.4.12.5). The certificate number shall have an X suffix in accordance with IEC 60079-0 and the specific conditions for use on the certificate shall draw the user’s attention to the requirement to check and retighten the seal compression element. |  |  |