- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE:  ExTAG/451/CD Draft ExTAG Decision Sheet – Application of IEC 60079-28

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

Draft Decision Sheet ExTAG/373A/CD, Scope exception regarding LED light sources was prepared and issued to Members for comment in June and August 2015, with all comments received addressed in ExTAG/395/CC.

During the September 2015 meeting of ExTAG in Christchurch, these documents were discussed, with this discussion reported on under Topic 10.9 of ExTAG/413/R, Report of the Seventeenth Meeting of the ExTAG- Christchurch, New Zealand.  As requested by ExTAG, subsequent to the meetings in Christchurch, the Convenor of IEC/TC 31 MT 60079-28 was contacted and concurrence was received regarding ExTAG/373A/CD.

Prior to adoption of ExTAG/373A/CD, additional ExTAG questions were raised, and this topic was included for discussion at the ExTAG training session during the September 2016 meeting of ExTAG in Umhlanga..

Draft Decision Sheet ExTAG(Umhlanga/Sec)05 was prepared following the ExTAG Training Session and issued for comment and discussion the following day during the September 2016 meeting of ExTAG.  This green paper took into account the extensive discussion from the training session which was guided by Draft Decision Sheet ExTAG/409/CD and the related comments document ExTAG/431/CC along with the presentation from Xu Jiang Ping which is available to download in the “presentations” section of the meeting papers.

As agreed at the meeting, IEC TC31 WG22 was consulted by preparing a comment on clause 6.6 of the CDV text of the proposed Edition 7 of IEC 60079-0.  WG22 did not believe it was appropriate to put the full proposal into the standard, but accepted that a clause was necessary in order to draw attention to the fact that IEC 60079-28 applies to LED light sources, in addition to the sources of radiation mentioned in the title of the document.  It was also noted that this new sub-clause will act as a place marker in IECEx reports for stating whether IEC 60079-28 has been applied or not, and for a “Not applicable” comment to state the reason it is not applicable.

This Draft Decision Sheet ExTAG/451/CD has been prepared taking into account all the above and is circulated for final comment and vote by correspondence.

Please submit comments using the comments table, a separate document by -

2017 05 18 to

Christine Kane

If there are no comments received the document will be forwarded for publication.

Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-0:2017 (Ed 7.0 at FDIS) IEC 60079-0:2011 (Edition 6.0) IEC 60079-0:2007 (Edition 5.0) IEC 60079-28:2015 (Edition 2.0) IEC 60079-28:2006 (First Edition), including I-SH 01 | Clause: 6.6.4 6.6.2 6.6.2 1 1 |  |
| --- | --- | --- |
| Subject: Applicability of IEC 60079-28, especially regarding LED light sources Status of document: Draft | Key words: - Optical radiation - Light-emitting diode - LED - Luminaire | - Date: 2015 08 28 Originator of original proposals: UL LLC, SGS Baseefa, IECEx Secretariat, IEC TC31 WG22 Date: 2017 03 30 Originator of this text Ron Sinclair, on behalf of WG22, taking account of previous proposals TC/SC involved: IEC/TC 31 WG 22, IEC/TC 31 MT 60079-28 |
| Background Various interpretations are being made by IECEx ExCB and ExTL staff regarding the applicability of IEC 60079-28 in the context of clause 6.6.2 of IEC 60079-0:2011. This is further complicated in that not all ExCBs and ExTLs have IEC 60079-28 in their IECEx scope. Many of the problems are related to the fact that clause 6.6.2 of IEC 60079-0:2011 only refers to IEC 60079-28 in a note, and therefore there is the possibility that the note may be disregarded and IEC 60079-28 is not applied in all situations where it is relevant. The potential confusion is compounded by the wording of the Scope exceptions of IEC 60079-28. It is also noted that the Scope of the first and second editions of IEC 60079-28 are different. Accordingly, IECEx asked the maintenance teams for IEC 60079-0 and IEC 60079-28 to review the relevant text in both documents to ensure that the requirements for the application of IEC 60079-28 are absolutely clear. IEC TC31 WG22 responded at its meeting during March in Sydney and has included an additional clause in the FDIS text of Edition 7 of the standard which is expected to be published in late 2017. 6.6.4 Lasers, luminaires, and other non-divergent continuous wave optical sources The requirements for lasers, luminaires, and other non-divergent continuous wave optical sources such as LED luminaires, torches, and optical fibre transmitters / receivers are contained in IEC 60079 28. This ExTAG Decision Sheet is intended as an interim measure to ensure all ExCBs and ExTLs operate on the same basis until Edition 7 of IEC 60079-0 is published and to ensure that all ExCBs and ExTLs fully complete the ExTR in respect of Clause 6.6.4 of the new edition. Question: When should the requirements of IEC 60079-28 be applied to Ex Equipment, especially regarding LED light sources? Answer: . The risk of ignition due to optical radiation hazards shall always be addressed. The first consideration to be made is regarding the Scope exclusions in IEC 60079-28. Regarding LED light sources in a luminaire, such can only be excluded from IEC 60079-28 using the Scope exclusion in respect of EPLs Gc and Dc, Where none of the Scope exclusions in IEC 60079-28 apply, then the Ignition Hazard Assessment (IHA) process described in the normative Annex C to IEC 60079-28:2015 (as referenced by Clause 4, “General requirements”) shall be applied to all equipment containing a light source, including luminaires, laser devices, etc. The result of this IHA shall be recorded on the ExTR report format for IEC 60079-28 at clause 4. The indication of the application of, and compliance with, the “op is”, “op pr” or “op sh” requirements is suitable IHA documentation. In order to make it absolutely clear to the market place that the applicability of IEC 60079-28 has been considered, all equipment involving optical radiation (whether in or out of the scope of IEC 60079-28) shall comply with one of the following: - Include the full reference to IEC 60079-28 on the second sheet of the certificate, incorporating the required marking into the marking string; or - Include the following text in the “Equipment” field of the certificate: “The optical radiation output of the apparatus with respect to explosion protection is covered in this certificate based on Exception [X] to the scope of IEC 60079-28:2015.” Note: The reference to [X]” is to be replaced by the actual Exception 1), 2), 3), 4) or 5) that applies from IEC 60079-28:2015. Where option 2 has been applied, and an ExTR report section for IEC 60079-28 is not included in the ExTR report, the record of the decision, with any necessary justification, shall be recorded in the ExTR report section for IEC 60079-0 at clause 6.6.2. Additional information: ExTLs and ExCBs do not need to have IEC 60079-28 in their IECEx Scope in order to use Option 2. It has been found that manufacturers of luminaires are not always aware of IEC 60079-28 and that they have submitted luminaires (particularly of Ex d construction) where the use of high power LEDs mounted close to the front glass has caused the luminaire to fail the test requirements of IEC 60079-28. |  |  |