- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE: Draft ExTAG Decision Sheet – Proposed Revision of ExTAG DS 2015/015 Enclosures welded or brazed together

Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, ExTAG/455/CD Draft ExTAG Decision Sheet – Proposed Revision of ExTAG DS 2015/015 Enclosures welded or brazed together, was prepared by UL LLC, US.

It is issued for a six week comment period for consideration by ExTAG Members.

Please submit comments using the comments table, a separate document by -

2017 08 04 to

Christine Kane

If there are no comments received the document will be forwarded for publication.

Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

DS 2015/015A

June 2017

Replaces DS 2015/015

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-1 Ed. 7 (2014) IEC 60079-1 Ed. 6 (2007) | Clause: 3.3, 6.1, 16.2, 16.3 (Ed. 7) 3.3, 6.1, 16.2 (Ed. 6) | # Date: 2017 06 09 Originator of proposal: UL LLC |
| --- | --- | --- |
| Subject: Enclosures welded, soldered, or brazed together Status of document: Draft | Key words: Welding, brazing, soldering, joint, enclosure | TC/SC involved: TC 31 MT 60079-1 Draft Decision Sheet: ExTAG/350B/CD |
| Title: Brazing, soldering and welding of flameproof enclosures. Background: Some flameproof enclosures are made from multiple pieces that have been welded together, due to size. Some enclosures are made from multiple pieces that have been brazed or soldered together, for the same reason. Although neither IEC 60079-0 nor IEC 60079-1 defines welding, soldering , or brazing, the practices are generally understood. IEC 60079-1 does have some provisions for welding (16.2 in Ed. 6, 16.2 and 16.3 in Ed. 7). Question 1: Is an enclosure section made up by welding pieces together considered a single enclosure section, or two sections with a flameproof joint? Question 2: Is an enclosure section made up by brazing pieces together considered a single enclosure section, or two sections with a flameproof joint? Question 3: Is an enclosure section made up by soldering pieces together considered a single enclosure section, or two sections with a flameproof joint? Answer: Question 1: A welded enclosure section is a single section, subject to the applicable type tests and routine tests. Question 2: A brazed enclosure section is a single section, subject to the applicable type tests and routine tests. Question 3: A soldered enclosure section is a single section, subject to the applicable type tests and routine tests. |  |  |