- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE:  Revised Draft Decision Sheet - The spark ignition test with super capacitor

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, ExTAG/478A/CD Revised Draft ExTAG Decision Sheet – The spark ignition test with super capacitor, has been prepared by the originator, PCEC, CN, and is circulated to ExTAG for comment.  The Compilation of Comments, along with observations from the originator, are contained in ExTAG/495/CC.

This updated version, ExTAG/478A/CD Revised Draft ExTAG Decision Sheet – The spark ignition test with super capacitor, takes into account the comments received.

In accordance with OD 035 it is issued for a six week comment period.

Please submit comments using the comments table, a separate document by

2018 04 20 to

Christine Kane

Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-11:2011 (Edition 6.0) | Clause: 7.1 10.1.4.2 |  |
| --- | --- | --- |
| Subject: The spark ignition test with supercapacitor Status: Draft | Key words: - Spark ignition test - Supercapacitor | Date: 2018 03 06 Originator of proposal: PCEC TC/SC involved: |
| Background: In IEC 60079-11:2011(Ed 6) clause10.1.4.2 a2) “Alternatively when an infallible current-limiting resistor is used with a capacitor, consider the capacitor as a battery and the circuit as resistive.“ Question: 1. May supercapacitors be used in intrinsically safe equipment, and if so how should they be assessed? 2. If a resistor is required to limit the output of the supercapacitor, how should the resistor be rated? Answer: 1. Yes, electrochemical capacitors (including supercapacitors, ultracapacitors and electric double layer capacitors) may be used in intrinsically safe equipment. When applying the requirements of IEC 60079-11 Ed. 6, they should be treated as cells or batteries rather than capacitors. 2. A resistor relied upon to limit the output of the supercapacitor shall be rated using the worst case voltage present on the supercapacitor. For its power rating, according to clause 7.1 of IEC 60079-11 Ed. 6, 1.5CU2 should be used. |  |  |