INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

TITLE:  Compilation of comments and observations on ExTAG/531/CD - Draft ExTAG Decision Sheet – re-non-closely coupled single packaged opto-isolators

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document contains the compilation of comments received on ExTAG/531/CD - Draft ExTAG Decision Sheet – re-non-closely coupled single packaged opto-isolators, with observations from the originator, MSTC, AU.

As a result of comments received and considered, a revised Draft Decision Sheet ExTAG/531A/CD has now been published for consideration and is issued for additional consideration over a six week period.

- Please inform the Secretariat immediately of any omissions or errors at

- Christine Kane

On behalf of Mr. Julien Gauthier

- Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
| DEKRA Certification B.V. |  |  | G | Understand the situation, however, in principle sufficiently mentioned in the standard. We do not support the DS, the IEC 60079-11 is under construction and this can be better worded in the new edition | Withdraw the DS | In the meantime (till the issue of IEC 60079-11 Ed 7), it would be beneficial to have a decision sheet that allows all ExTLs and ExCBs to have the same definition of what is a closely coupled device and what is not |
| FME (GB) |  |  | G | IEC 60079-11 does not define what it means by the term close-coupled and this term is only used in a note. One definition for ‘close-coupled’ is ‘(of two parts of a structure) attached or fixed close together”, and on this basis since the item mentioned in the draft DS is a single package devices the requirements of 10.11 apply. However, we are not sure if this was the intent of the MT when this section was written. | We suggest that this is pushed to MT60079-11 for an interpretation sheet on the term ‘close-coupled’ as this relates to items such as opto-couplers. | Because the intent of introducing the “close-coupled” term is not known, a DS would give a unified interpretation. |
| LOM ES |  |  | T | LOM agrees the DS as long as there was not a proposal from the MT saying that it is not “close coupled”. | Include that it is not a “close coupled” device. It is a device for which the inner and outer separation distances have been measured and meet with table 5 or Annex F. The external material comply with the required CTI values. The inner and outer measurements have been determined assessing 10 samples in accordance with 10.4 of IEC 60079-11. Ed.6 | Thank you. A revised proposal is being submitted to include a more accepted proposal. |
| NANIO CCVE (ExCB and ExTL) RU |  |  | General | We support this Decision Sheet without comments |  | Thank you. A revised proposal is being submitted to include a more accepted proposal. . |
| NCC BR |  |  |  | NCC supports ExTAG/ 531/CD |  | Thank you. A revised proposal is being submitted to include a more accepted proposal. |
| NEPSI CN |  |  | G | We support the draft DS ExTAG/531/CD. |  | Thank you. A revised proposal is being submitted to include a more accepted proposal. |
| TIIS JP |  |  | Editorial | Generally we agree with the proposed DS. We think that it is not necessary even for a single package to conform to clause 7.1 as long as the internal separate distance of the component complies with relating requirements. | Answer should be changed to: In the case of conformity to Table 5 including columns 5, 6 and 7, or column 2 of Table F.1, the requirements of clause 10.11 do not apply. | Thank you. A revised proposal is being submitted to include a more accepted proposal. |
| UL BR |  |  | Technical | It is agreed that a better definition of a “close-coupled” would be beneficial, however the proposed wording is in conflict with the requirement in a) of 8.9.2 which requires compliance with Table 5 or Table F.1. Concerning the question, it is already stated in the note that the tests of 10.11 are only intended to apply to close-coupled devices. As discussed in ExTAG, this should be handled as an ISH rather than a DS. | Where a device is a single package, but has been designed to have a large comply internally with column 5 and 7 distance of (exceeding Table 5 requirements) between the transmitter and receiver, then it is not a close-coupled device. In such case, the requirements of Clause 10.11 do not apply. | Thank you. A revised proposal is being submitted which I believe reflect your proposal. |
| UL US |  |  | Technical | It is agreed that a better definition of a “close-coupled” would be beneficial, however the proposed wording is in conflict with the requirement in a) of 8.9.2 which requires compliance with Table 5 or Table F.1. Concerning the question, it is already stated in the note that the tests of 10.11 are only intended to apply to close-coupled devices. As discussed in ExTAG, this should be handled as an ISH rather than a DS. | Where a device is a single package, but has been designed to have a large comply internally with column 5 and 7 distance of (exceeding Table 5 requirements) between the transmitter and receiver, then it is not a close-coupled device. In such case, the requirements of Clause 10.11 do not apply. | Thank you. A revised proposal is being submitted which I believe reflect your proposal. |