COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-1:2014 (Ed 7) | Clauses: Annex G | Decision Sheet: ExTAG/538A/CD |
| --- | --- | --- |
| Subject: Construction requirements for flameproof enclosures with an internal source of release (containment system) Status of document: Approved | Key words: - Containment system | - Date: 2019 05 Originator of proposal: FM Approvals LLC (FMG) TC/SC involved: IEC/TC 31/MT60079-1 |
| Question: Can the tests given in G.4 of IEC 60079-1, Ed 7, be omitted if the containment system inside flameproof enclosure complies with the burst pressure test in 5.2.5 of IEC TS 60079-40 followed by the Helium leak tests in 4.1.3.2 of IEC 60079-26? Answer: Yes. A containment system that meets the design requirements of G.3 of 60079-1 along with the pressure test in 5.2.5 of IEC TS 60079-40, using a test time of at least 2 minutes at a minimum test pressure of 1 000 Pa, followed by the Helium leak tests in 4.1.3.2 of IEC 60079-26 would provide appropriate justification for the tests of G.4 to be omitted. If non-metallic materials other than glass or ceramic are used to effect the sealing of the containment system, it is noted that thermal endurance preconditioning is also required prior to the burst pressure and Helium leak tests. |  |  |