COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-15:2017 (Ed 5.0) IEC 60079-15:2010 (Ed 4.0) | Clauses: 9.2, 3.1.3 19.2, 3.7.2.4 | Decision Sheet: ExTAG/561/CD |
| --- | --- | --- |
| Subject: Opening of Sealed devices on frequent basis (e.g. for charging) Status of document: Approved | Key words: - Sealed - Opening - Frequent - Charging - Normal operation - Normal service | - Date: 2019 07 23 Originator of proposal: UL LLC TC/SC involved: IEC/TC 31 MT60079-15 |
| Background: IEC 60079-15 requires that “Sealed devices shall be so constructed that they cannot be opened in normal operation”. Further, it defines a Sealed device in part as a “device which is so constructed that it cannot be opened during normal service…”. There are differing opinions between certifiers regarding when a Sealed device can be opened in accordance with these requirements. For example, there are some that consider the frequent opening of a Sealed device for an activity such as charging is permitted, as long as the opening and charging occurs outside of the explosive atmosphere. Question: If the housing of a Sealed device is required to be opened, such as for charging, would such be permitted even if the device complies with all other requirements for a Sealed device? Answer: No. The evaluation of a Sealed device does not address the potential adverse effect that frequent opening and closing has on the Sealed device (such as on a gasket). A visual examination of the parts of the seal effected by the opening and closing would likely not be enough to assure that the Sealed device was still the same as when the device was originally assembled. The opening/closing of the Sealed device must only be considered as part of a maintenance function, where the activity is controlled and assurance is made that the Sealed device maintains the integrity to seal as originally assembled. |  |  |