- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Title:  ExTAG/569/CD - Draft ExTAG Decision Sheet – Thermal endurance tests of large enclosures

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, ExTAG/569/CD Draft ExTAG Decision Sheet - Thermal endurance tests of large enclosures has been prepared by Laboratorio Oficial Jose María de Madariaga, LOM, ES, and is issued for consideration by ExTAG.

In accordance with OD 035 this document is issued for a six week comment period.

Please submit comments on this new Draft DS using the comments table, a separate document, by –

2019 09 06

to

Christine Kane

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-0:2011 Edition 6 IEC 60079-0:2017 Edition 7 | Clause: 26.8 26.9 |  |
| --- | --- | --- |
| Subject: Thermal endurance tests of large enclosures Status: Draft | Key words: - Thermal endurance tests - Enclosures | Date: 2019 07 26 Originator of proposal: LOM TC/SC involved: TC 31 (MT 60079-0) |
| Background: When enclosures are subjected to thermal endurance cycles and they are very tight, depressions occur inside. If the enclosures are relatively large, it can result in deformations of the walls that can lead to abnormal defects of the gaskets. When this event occurs in the industry, the enclosures are provided with breathing devices. Either an empty enclosure or an enclosure equipped with cable entries should be tested under the most onerous conditions (clause 26.2 in both editions of IEC 60079-0). This involves closing any type of hole tightly, either the cable entries or holes for further tests such as IP or pressure tests. Question: Should the enclosure be subjected to thermal endurance tests without additional holes? Answer: Yes. In case of being necessary to install breathing devices to be able to overcome the tests the certificate shall bear a special condition of installation. |  |  |