- INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Title:  ExTAG/536B/CD Revised Draft ExTAG Decision Sheet - Suffix “X” and “U” to the certificate.

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document, ExTAG/536B/CD Revised Draft ExTAG Decision Sheet Decision Sheet Suffix “X” and “U” to the certificate has been prepared by SITIIAS/NEPSI, CN, taking into account comment received on ExTAG/536A/CD, which it now replaces.

ExTAG/536B/CD is issued, including track changes, in accordance with OD 035, for additional consideration over a six week period.

A Compilation of Comments on ExTAG/536A/CD along with originator observations are contained in ExTAG/571/CC.

Please submit comments using the comments table, a separate document by –

2019 09 09

Christine Kane

Julien Gauthier

Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-0:2011 IEC 60079-0:2017 IEC 60079-7:2015 | Clauses: 3.52, 3.53, 13,5, 29.9, 3.36, 3.83, 13.5, 3.84, 28.2 4.10 |  |
| --- | --- | --- |
| Subject: Suffix “X” and “U” to the certificate Status of document: Draft | Key words: Ex component Suffix “X”, “U” | # Date: July 24, 2018 July 2019 Originator of proposal: SITIIAS/NEPSI |

Background:

With the extensive application of fieldbus, process control system adopts more and more power supply modules, remote IO modules and input/output modules etc.. It is popular that those modules have to be mounted into an additional housing and tested according to IEC 60079-0 and IEC 60079-7 (former IEC 60079-15) as Ex Components. So many certificates, with either suffix “X” or “U”, have been issued already for those types of modules. It looks that each certification body have individual rules to apply suffix “X” or “U”.

QUESTION: For Ex ec modules (such as power supply modules, remote IO modules and input/output modules), intended to be mounted completely within a separate Ex ec enclosure, shall suffix “U” or “X” be applied to the certificate for the module?

ANSWER:

Suffix “ U” shall be marked on the those modules, when they are evaluated individually and the temperature rise (both service and surface) will need to be determined after installation in a suitable end product enclosure.

IEC 60079-0:2017 clause 3.84 has described that suffix “X” on the certificate number denotes Special Condition of Use for Ex Equipment. IEC 60079-0:2017 clause 3.83 has described that suffix “U” on the certificate number identifies for the module as  Ex Components, which is incomplete equipment and not suitable for installation without further evaluation.

[Explanation] These Ex ec modules are intended to be mounted into an Ex ec enclosure. The ingress of protection (IP) of the enclosure shall be tested according to IEC 60079-0 and IEC 60079-7 as the IP requirements in the general industrial standards applied, would not include the additional requirements for explosion protection (IEC 60079-7:2015 clause 4.10.1 note). The mounting of modules will need to be further evaluated for the separation distances and temperature rises etc..  All of these are critical factors for complete Ex equipment (enclosure & modules). So it is reasonable to define those modules as Ex Components, and suffix “U” shall be applied to the IECEx certificate number.