INTERNATIONAL ELECTROTECHNICAL COMMISSION SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- TITLE:  Compilation of comments and observations on ExTAG/565/CD - Draft ExTAG Decision Sheet – IP testing of cable entry devices

- Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document contains the compilation of comments received, as well as observations from the originator, SGS Baseefa, GB, on ExTAG/565/CD - Draft ExTAG Decision Sheet – IP testing of cab

As a result of comments received, and considered, Decision Sheet ExTAG DS 2019/005 has now been published and is available from the IECEx Web Site.

- Please inform the Secretariat immediately of any omissions or errors at-

- Christine Kane

On behalf of Mr. Julien Gauthier

- Julien Gauthier

ExTAG Secretary

| Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com | ExTAG Secretary Mr Julien Gauthier LCIE S.A. 33 Avenue du General Leclerc 92260 Fontenay-aux-Roses FRANCE Tel: +33 1 40 95 55 26 Fax: +33 1 40 95 89 37 Email : julien.gauthier@fr.bureauveritas.com |
| --- | --- |

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | All contributors are unanimous in choosing Answer 2 The DS is therefore revised to show answer 2 only, plus additional text reflecting the view of TC31 WG22 re the application of the instructions. TC31 WG22 is requested to consider if an I-SH is needed |
| DEKRA / BVS DE |  |  |  | We support answer 2 |  |  |
| DEK NL |  |  |  | Answer 2 meets our understanding of the intent of the standard |  |  |
| ExTC GB |  |  | General | We concur with Proposed Answer 2 | We concur with Proposed Answer 2 |  |
| Kiwa NL |  |  | G | Kiwa prefers proposed answer # 2 | Remark: the thermal endurance tests do not apply for metallic sealing devices. The sealing device shall also comply with the requirements of clause 7 and the details shall be included in the technical documentation of the gland. |  |
| LCIE FR |  |  | General | this matter should be handled directly by the IEC/TC 31 (WG 22) through an ISH more than a IECEx DS. It is even written in the document "understanding of the intent of the standard". | transfer to IEC/TC 31 (WG 22) |  |
|  |  |  |  | we consider that answer 2 is the most appropriate |  |  |
| NANIO CCVE (ExCB and ExTL) RU |  |  | General | We accept the proposed answer 2 |  |  |
| NCC BR | A.3.4 |  |  | We agree with proposed answer 2. |  |  |
| NEPSI CN |  |  | G | We support the second choice of the proposed answers. |  |  |
| PTB DE |  |  | Technical | The interface between the cable gland and enclosure needs to be tested in the same manner as for other enclosure parts like covers or operators with their sealing devices. | Use proposed Answer 2 TC 31 WG22 is requested to address the matter of the instructions and compatibility with the related requirements in IEC 60079-14. |  |
| SIQ SI |  |  |  | We agree proposal number 2. |  |  |
| TC31/WG22 |  |  | te | The revisions with respect to cable gland entry sealing made to IEC 60079-0, Edition 7, were made to ensure that a completed installation complied with the required IP ratings for the various Types of Protection. Without addressing the sealing between the cable gland and the enclosure, there was no point in addressing the sealing of the cable gland as it could/would be compromised by the lack of or failure of the interface sealing. The intent of WG22 was certainly that the interface seal or seals be specified in the instructions and that any seal used for the testing be in accordance with the installation instructions. If this is not clear, the text can be revised in Edition 8 to include clarifying statements. | Agree with “Answer 2”. The DS could reiterate that the cable gland / enclosure test sample has to take into account the installation instructions. IEC 60079-0, Edition 7, Sub-Clause 26.2, requires that the test configuration take into account the instructions. The test program is of little value if this is not followed. IEC 60079-14, Edition 5, requires that the instructions be available and that they be applied. A certification, if obtained, would be of little value if the instructions were not applied. |  |
| TIIS JP |  |  | ge/te | TIIS supports the issuance of the DS with “Proposed answer 2”. |  |  |
| TUVN DE |  |  |  | TÜV NORD suggest that Answer 2 shall be mandatory for cable glands |  |  |
| TUV SUD PS DE | A.3.4 |  | Technical | We concur with proposed answer 2. Additional consideration should be given to the following: The sealing effectiveness between gland and enclosure does not only depend on the gland’s sealing method but also on the enclosure’s surface roughness or structure. Some sealing rings made of a relatively hard material can effectively seal unpainted (metal) enclosures having a smooth surface, however painted enclosures, especially enclosures with structured paint, cannot always effectively be sealed to pass the IP test. For structured paint enclosures a washer/sealing method using a softer material has to be chosen so that the washer adapts to the surface structure of the enclosure to effectively seal the interface between gland and enclosure. |  |  |
| UL USA |  |  |  | For 565/CD, UL-USA agrees with proposed answer #2. |  |  |
| ULBR BR |  |  | General | ULBR agrees with proposed answer #2. |  |  |
| ULD DK |  |  | General | We agree with proposed answer #2. |  |  |