INTERNATIONAL ELECTROTECHNICAL COMMISSION (IEC) SYSTEM FOR CERTIFICATION TO STANDARDS RELATING TO EQUIPMENT FOR USE IN EXPLOSIVE ATMOSPHERES (IECEx SYSTEM)

- Title: Compilation of Comments on ExTAG/569/CD- Draft ExTAG Decision Sheet – Thermal endurance tests of large enclosures

Circulated to: ExTAG – IECEx Testing and Assessment Group

INTRODUCTION

This document contains the Compilation of Comments and Observations from the Originator Laboratorio Oficial Jose María de Madariaga, LOM, ES, on ExTAG/569/CD- Draft ExTAG Decision Sheet – Thermal endurance tests of large enclosures

- Please inform the Secretariat immediately of any omissions or errors at

- Christine Kane

Address: IECEx Secretariat Level 33 Australia Square 264 George Street Sydney NSW 2000 Australia Web: www.iecex.com

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
| CML GB |  |  | TE | The temperature change in service is not likely to be instantaneous, such as when the environmental chamber door is opened. Although temperature may vary significantly over 24h, the DS is suggesting all enclosures should be tested for thermal shock. The test is to simulate ageing, not thermal shock. | We consider the answer should be no. | Not accepted. For clarity, it must be assumed that all ExTLs conduct thermal endurance testing avoiding sudden temperature changes. Additionally, the purpose of this DS is to unified the criteria when leaving a cable entry open have a big impact on the test result, especially when testing large enclosures |
|  |  |  |  | To avoid deformation of the enclosure by sudden temperature change it is best to leave the enclosure in the chamber and just allow it to cool slowly, without opening the door. This is allowed in clause 26.8 with the +30h time tolerance. Alternatively, leave a cable entry hole open. | No additional advice necessary. |  |
| CNEX-Global BV NL | - | - | G | We agree with this DS | None | Noted |
| DEK NL |  |  | GE | We disagree with the DS, it is adding requirements, which is not allowed per OD 035 | Withdraw DS | Not accepted. Refer to Clause 26.2 of IEC 60079-0 in Ed.6 and Ed.7. No requirements are added. |

| ExCB/ ExTL | Clause/ Sub-clause | Paragraph Figure/ Table | Type of comment General/ technical/ editorial | COMMENTS | Proposed change | Observation (to be completed by the originator) |
| --- | --- | --- | --- | --- | --- | --- |
| DNV Presafe NO |  |  | General | It is not clear what situations the DS is trying to address. Is it problems with gaskets supplied with the UUT or distortion that occurs in the test that might make seals on glands and entry devices that are added by the user fail a test? | We suggest that the originator work with maintenance team to clarify this. | Not accepted. The background will be modified to a better understanding on what the DS is trying to address. |
| NANIO CCVE (ExCB and ExTL) RU |  |  | General | The logic of the question and answer is not clear. If a breathing hole is provided on the enclosure, it cannot be considered tight. |  | Not accepted. The answer does not include the wording ‘breathing hole’. A breathing device with the same or greater degree of protection shall be used. |
| NCC BR | 26.8 26.9 |  |  | We agree. |  | Noted |
| PTB DE | Answer |  | g | To avoid any misunderstanding, it should be clear that Breathing and draining devices needs to be evaluated together with the enclosure (even if the draining device itself is certified as a components). | Yes. In case of any special measurers (e.g. the requirements of a breathing devise) to withstand the thermal endurance and IP test, these measures need to be described in detail in the schedule drawings. Furthermore, it might be necessary to specify special conditions. | Accepted in principle |
| TC31 |  |  | GE | It is not clear what situations the DS is trying to address. In practice, a manufacturer of a box or Ex rated equipment in a box submits his item to a TL for test. Any entries supplied with the unit are plugged for the test. If the box is sold with no holes and they are subsequently added by the installer, the TL does not add holes for the test. If gaskets supplied with the UUT by the manufacturer get distorted such that the box needs a breather, the breather should be part of the product supplied for the test and included with the box to the customer. If the text is referring to distortion that occurs in the test that might make seals on glands and entry devices that are added by the user fail a test, that is a different situation, but it is not clear what gaskets they are trying to cover here. | We request that the originator withdraw the DS and work with the WG22 Convenor and TC31 Liaison to clarify what we are trying to address. | Noted The DS is trying to address what it is said in second paragraph of TC31 comment. The aim is to unified the criteria to avoid the situation in which large enclosures pass thermal endurance tests in an ExTL because holes where not closed and the same enclosures do not pass thermal endurance tests in other ExTL just because all open holes where plugged. This case comes a manufacturer having this situation with two different ExTLs and unification of criteria on this regard should be addressed. |
| TIIS JP |  |  | te | Users can not use the breathing devices which are not tested. Therefore, there should not be a certificate with specific conditions of use with those devices. When it is necessary to install breathing devices to the enclosure, type tests should also be carried out with breathing devices. | Change the sentence after “Yes” in Answer as follows: In case of being necessary to install breathing devices to be able to overcome the tests, type tests should be carried out with the breathing device(s). | Accepted in part The wording after “Yes” will be modified. |
| TUV SUD PS DE | 26.8 26.9 |  | General | We agree with the decision. | - | Noted |
| UL- USA | Answer |  | General | The proposal is unclear and seems to be introducing test conditions that may be more severe than what is intended by the standard. | Refer this topic to the TC31 MT. Do not issue the Decision Sheet. | Not accepted. Refer to Clause 26.2 of IEC 60079-0 in Ed.6 and Ed.7. |
| UL BR |  |  | General | The proposed changes the requirements in the standard and should be referred to the MT. |  | Not accepted |