COLLECTION OF IECEx / ExTAG DECISION

| Standard: IEC 60079-11:2011 (Edition 6.0) IEC 60079-11:2006 (Edition 5.0) | Clause: 7.5 7.5 |  |
| --- | --- | --- |
| Subject: Software dependent semiconductor components Status of document: Approved | Key words: - Semiconductor - Microcontroller - Protective component - Active voltage limiting circuit - Active current limiting circuit - Software | - Date: 2015 08 11 Originator of proposal: UL LLC TC/SC involved: IEC/SC 31G MT 60079-11 Original Draft: ExTAG/370/CD |
| Question Can software dependent components (for example microcontrollers), operating independently and meeting the requisite requirements for redundancy, be used as protective components in either active voltage or active current limiting circuits? Note – for the purposes of this DS, “software” means either software or firmware. Answer No. While reference may be made to “controllable semiconductor components”, this does not mean software controllable. The use of software dependent components would necessitate establishing an appropriate SIL for the software and conducting an evaluation of the software even though the microcontrollers are acting independently. As an appropriate SIL has not been defined in 60079-0 or 60079-11, this would be outside the scope of the standards and therefore cannot be used as protective components. |  |  |