COLLECTION OF IECEx / ExTAG DECISIONS

| Standard: IEC 60079-7 | Clauses: - - 4th Edition 4.7.1.2 – 3rd Edition |  |
| --- | --- | --- |
| Subject: Determining compliance with winding requirements for enameling Status of document: Approved | Key words: - Winding - Wire - Enamel - Grade | # Date: 2010-11-09 Originator of proposal: UL TC/SC involved: IEC/TC31 |
| Question: If a wire manufacturer’s specification is provided that specifies the enamel grade of a winding in accordance with the applicable IEC standard (i.e. IEC 60317-3, IEC 60317-7, IEC 60317-8, or IEC 60317-13), is it required that the ExTL and ExCB verify conformity of the winding to the manufacturer’s specification regarding the enamel grade? Answer: No, it is not a requirement of this standard that conformity to the manufacturer’s specification regarding the enamel grade of the winding needs to be verified. Conformity can be determined based on the manufacturer’s specification. |  |  |